ALU (4 Bit) [Behavioural]

Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

use ieee.std_logic_unsigned.all;

entity ALU_4bit is

Port ( A : in STD_LOGIC_VECTOR(3 downto 0);

B : in STD_LOGIC_VECTOR(3 downto 0);

S : in STD_LOGIC_VECTOR(2 downto 0);

Y : out STD_LOGIC_VECTOR(3 downto 0));

end ALU_4bit;

architecture Behavioral of ALU_4bit is

begin

process(S)

begin

if S = "000" then

Y <= A + B;

elsif S = "001" then

Y <= A - B;

elsif S = "010" then

Y <= A + 1;

elsif S = "011" then

Y <= B + 1;

elsif S = "100" then

Y <= A AND B;

elsif S = "101" then

Y <= A OR B;

elsif S = "110" then

Y <= A NAND B;

elsif S = "111" then

Y <= A NOR B;

end if;

end process;

end Behavioral;

TestBench:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

use ieee.std_logic_unsigned.all;

entity ALU_TB is

end ALU_TB;

architecture Behavioral of ALU_TB is

component ALU_4bit

Port (

A : in STD_LOGIC_VECTOR(3 downto 0);

B : in STD_LOGIC_VECTOR(3 downto 0);

S : in STD_LOGIC_VECTOR(2 downto 0);

Y : out STD_LOGIC_VECTOR(3 downto 0));

end component;

signal A: std_logic_vector(3 downto 0);

signal B: std_logic_vector(3 downto 0);

signal S: std_logic_vector(2 downto 0);

signal Y: std_logic_vector(3 downto 0);

begin

uut: ALU_4bit port map (A, B, S, Y);

process

begin

A <= "0111";

B <= "0011";

S <= "000";

wait for 10 ns;

S <= "001";

wait for 10 ns;

S <= "010";

wait for 10 ns;

S <= "011";

wait for 10 ns;

S <= "100";

wait for 10 ns;

S <= "101";

wait for 10 ns;

S <= "110";

wait for 10 ns;

S <= "111";

wait for 10 ns;

wait;

end process;

end Behavioral;