Counter 4-bit

Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity Counter_4 is

port(

rst : in  STD_LOGIC;

pr :  in STD_LOGIC;

clk : in  STD_LOGIC;

clk_div : inout STD_LOGIC;

dir : in  STD_LOGIC;

Q : out  STD_LOGIC_VECTOR (3 downto 0)

);

end Counter_4;

architecture Behavioral of Counter_4 is

signal Qtemp : STD_LOGIC_VECTOR (3 downto 0) := "0000";

signal Counter : STD_LOGIC_VECTOR (26 downto 0) := (others => '0');

begin

process(rst,pr,clk_div,dir)

begin

if rst ='1' then

Qtemp <= (OTHERS =>'0');

elsif pr='1' then

Qtemp <= (OTHERS =>'1');

elsif falling_edge(clk_div) then

if dir = '1' then

if Qtemp < 15 then

Qtemp <= Qtemp + 1;

else

Qtemp <= "0000";

end if;

else

if Qtemp > 0 then

Qtemp <= Qtemp - 1;

else

Qtemp <= "1111";

end if;

end if;

end if;

end process;

Q <= Qtemp;

process(clk)

begin

if clk'event and clk = '1' then

counter <= counter + 1;

end if;

end process;

clk_div <= counter(0);

end Behavioral;

TestBench:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity Counter_4_TB is

end Counter_4_TB;

architecture Behavioral of Counter_4_TB is

component Counter_4 is

port(

rst : in  STD_LOGIC;

pr :  in STD_LOGIC;

clk : in  STD_LOGIC;

clk_div : inout std_logic;

dir : in  STD_LOGIC;

Q : out  STD_LOGIC_VECTOR (3 downto 0)

);

end component;

signal rst : std_logic := '0';

signal pr  : std_logic := '0';

signal clk : std_logic := '0';

signal clk_div : std_logic := '0';

signal dir : std_logic := '0';

signal Q : std_logic_vector(3 downto 0);

begin

uut: Counter_4 port map(rst, pr, clk, clk_div, dir, Q);

clk_process : process

begin

clk <= '0';

wait for 10 ns;

clk <= '1';

wait for 10 ns;

end process;

stim_proc : process

begin

rst <= '1';

wait for 20 ns;

rst <= '0';

--Up Counter

dir <= '1';

wait for 500 ns;

pr <= '1';

wait for 20 ns;

pr <= '0';

--Down Counter

dir <= '0';

wait for 300 ns;

wait;

end process;

end Behavioral;

Constraints:

set_property PACKAGE_PIN L1 [get_ports {Q[3]}]

set_property PACKAGE_PIN P1 [get_ports {Q[2]}]

set_property PACKAGE_PIN N3 [get_ports {Q[1]}]

set_property PACKAGE_PIN P3 [get_ports {Q[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {Q[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {Q[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {Q[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {Q[0]}]

set_property PACKAGE_PIN W5 [get_ports clk]

set_property PACKAGE_PIN U16 [get_ports clk_div]

set_property PACKAGE_PIN R2 [get_ports dir]

set_property PACKAGE_PIN T1 [get_ports pr]

set_property PACKAGE_PIN U1 [get_ports rst]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports clk_div]

set_property IOSTANDARD LVCMOS33 [get_ports dir]

set_property IOSTANDARD LVCMOS33 [get_ports pr]

set_property IOSTANDARD LVCMOS33 [get_ports rst]

set_property CLOCK_DEDICATED_ROUTE FALSE [get_nets pr_IBUF]

MOD 25 Counter

Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity mod25 is

Port (

rst : in STD_LOGIC;

pr  : in STD_LOGIC;

clk : in STD_LOGIC;

clk_div : inout STD_LOGIC;

dir : in STD_LOGIC;

Q   : out STD_LOGIC_VECTOR(4 downto 0)

);

end mod25;

architecture mod25_arch of mod25 is

signal Qtemp : STD_LOGIC_VECTOR(4 downto 0) := "00000";

signal Counter : STD_LOGIC_VECTOR (27 downto 0) := (others => '0');

begin

process(rst, pr, clk, dir)

begin

if rst = '1' then

Qtemp <= (others => '0');

elsif pr = '1' then

Qtemp <= (others => '1');

elsif falling_edge(clk) then

if dir = '1' then  --UP counting

if Qtemp < 24 then

Qtemp <= Qtemp + 1;

else

Qtemp <= "00000";

end if;

else              --DOWN counting

if Qtemp > 7 then

Qtemp <= Qtemp -1;

else

Qtemp <= "11111";

end if;

end if;

end if;

end process;

Q <= Qtemp;

process(clk)

begin

if clk'event and clk = '1' then

counter <= counter + 1;

end if;

end process;

clk_div <= counter(0);

end mod25_arch;

Test Bench:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity mod25_tb is

end mod25_tb;

architecture Behavioral of mod25_tb is

--Component Declaration for the Unit Under Test (UUT)

component mod25

Port (

rst : in STD_LOGIC;

pr  : in STD_LOGIC;

clk : in STD_LOGIC;

clk_div : inout STD_LOGIC;

dir : in STD_LOGIC;

Q   : out STD_LOGIC_VECTOR(4 downto 0)

);

end component;

--Signals to connect to the UUT

signal rst : STD_LOGIC := '0';

signal pr  : STD_LOGIC := '0';

signal clk : STD_LOGIC := '0';

signal clk_div : STD_LOGIC := '0';

signal dir : STD_LOGIC := '1'; --default: up count

signal Q   : STD_LOGIC_VECTOR(4 downto 0);

begin

--Instantiate the Unit Under Test (UUT)

uut: mod25

port map (

rst => rst,

pr  => pr,

clk => clk,

clk_div => clk_div,

dir => dir,

Q   => Q

);

--Clock generation: 10 ns period (falling edge sensitive)

clk_process : process

begin

while true loop

clk <= '1';

wait for 5 ns;

clk <= '0';

wait for 5 ns;

end loop;

end process;

--Stimulus process

stim_proc: process

begin

--Initial reset

rst <= '1';

wait for 20 ns;

rst <= '0';

wait for 20 ns;

--Up count for a few cycles

dir <= '1';  --count up

wait for 300 ns;

--Now preset

pr <= '1';

wait for 20 ns;

pr <= '0';

wait for 20 ns;

--Down count

dir <= '0';

wait for 300 ns;

--Apply reset again

rst <= '1';

wait for 20 ns;

rst <= '0';

--End simulation

wait;

end process;

end Behavioral;

Universal Shit Register

Code:

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_unsigned.all;

entity USR_Code is

port (

clk : in std_logic;

clk_div : inout std_logic;

rst : in std_logic;

load : in std_logic;

mode : in std_logic_vector (1 downto 0);

si : in std_logic;

pi : in std_logic_vector (3 downto 0);

so : out std_logic;

po : out std_logic (3 downto 0)

);

end USR_Code;

architecture beh of USR_Code is

signal shift_reg : std_logic_vector(3 downto 0);

signal counter : std_logic_vector (26 downto 0) := (others => ‘0’);

begin

process(clk)

begin

if clk’event and clk = ‘1’ then

if rst = ‘1’ then

counter <= (others => ‘0’);

else

counter <= counter + 1;

end if;

end if;

end process;

clk_div <= counter(0);

process(clk_div, rst, load)

begin

if rst = ‘1’ then

shift_reg <= (others => ‘0’);

elsif

clk_div’event and clk_div = ‘1’ then

case mode is

when “00” =>

shift_reg(3 downto 1) <= shift_reg(2 downto 0);

shift_reg(0) <= si;

so <= shift_reg(3);

when “01” =>

shift_reg(3 downto 1) <= shift_reg(2 downto 0);

shift_reg(0) <= si;

po <= shift_reg;

when “10” =>

if load =’0’ then

shift_reg <= pi;

else

shift_reg(3 downto 1) <= shift_reg(2 downto 0);

end if;

so <= shift_reg(3);

when “11” =>

po <= pi;

when others => null;

end case;

end if;

end process;

end beh;

TestBench:

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_unsigned.all;

entity USR_TB is

end USR_TB;

architecture beh of USR_TB is

component USR_Code is

port (

clk : in std_logic;

clk_div : inout std_logic;

rst : in std_logic;

load : in std_logic;

mode : in std_logic_vector (1 downto 0);

si : in std_logic;

pi : in std_logic_vector (3 downto 0);

so : out std_logic;

po : out std_logic_vector (3 downto 0));

end com	ponent;

signal clk, clk_div, rst, load, si, so : std_logic := ‘0’;

signal mode : std_logic_vector(1 downto 0);

signal pi, po : std_logic_vector (3 downto 0);

begin

uut: USR_Code PORT MAP (clk, clk_div, rst, load, mode, si, pi, so, po);

clk_process : process

begin

clk <= ‘0’;

wait for 10 ns;

clk <= ‘1’;

wait for 10 ns;

end process;

stim_process : process

begin

rst <= ‘1’;

wait for 20 ns;

rst <= ‘0’;

mode <= “00”;

si <= ‘1’;

wait for 200 ns;

si <= ‘0’;

wait for 200 ns;

mode <= “01”;

si <= ‘1’;

wait for 100 ns;

si <= ‘0’;

wait for 200 ns;

mode <= “10”;

load <= ‘0’;

pi <= “1110”;

wait for 100 ns;

load <= ‘1’;

wait for 300 ns;

mode <= “11”;

pi <= “1010”;

wait for 50 ns;

pi <= “1010”;

wait for 50 ns;

wait;

end process;

end beh;